Perspective-Correct Quilt Synthesis from Integral Light Field Images

Technical Algorithm Documentation

June 2026

# Abstract

This document describes the algorithm implemented in integral_to_quilt_v3.py for converting a horizontal-parallax-only (HPO) integral light field image into a perspective-correct quilt suitable for a multi-view autostereoscopic display. Starting from the fundamental representation of the light field as a four-dimensional radiance function, we derive the closed-form formula that assigns the correct directional pixel index to each hogel in each quilt view. The derivation shows why the simpler "assembled view" approach — which samples the same directional index from every hogel — produces an orthographic rather than a perspective image, and how introducing a per-hogel parallax offset, computed from a pinhole camera model, restores correct perspective foreshortening. All parameter values are taken from a JSON display configuration file and carry their derivation explicitly.

# 1  The Light Field as a Four-Dimensional Radiance Function

The plenoptic function introduced by Adelson and Bergen describes the radiance arriving at a point in space as a function of direction, wavelength, and time. In a static scene under fixed illumination the spectral and temporal dimensions can be frozen, reducing the representation to a function of position and direction. In free space — regions containing no occluders — the radiance along a ray does not change; therefore the five-dimensional position-direction space collapses to a four-dimensional one, since any ray is fully described by the two coordinates of its intersection with each of two parallel planes.

Gershun coined the term light field for the radiometric description of light in a region of space. In the computer-graphics context, Levoy and Hanrahan and independently Gortler et al. formalised the four-dimensional light field and showed that new perspective views can be synthesised by extracting appropriate two-dimensional slices from it, without any depth information or feature matching.

# 2  The Two-Plane Parameterisation

Levoy and Hanrahan parameterise the light field using intersections of rays with two parallel planes. By convention, one plane — called the camera plane — is labelled (u, v) and a second focal plane is labelled (s, t). A ray is uniquely identified by the pair L(u, v, s, t), which gives the radiance of the ray connecting point (u, v) on the camera plane to point (s, t) on the focal plane.

Synthesising a new perspective view from position u0 on the camera plane amounts to fixing u = u0 and varying s over the full focal plane. Each pixel of the resulting view corresponds to one ray direction, and the radiance for that pixel is found by looking up the stored value L(u0, v0, s, t). When the camera plane is placed at infinity, all rays become parallel and the view is orthographic; when the camera plane is at a finite distance, the view is a perspective projection centred on the camera position.

The integral image studied here is a sampled light field in which the camera plane corresponds to hogel positions (the s-axis in the display plane) and the focal plane corresponds to the directional pixel axis. Each sample L(hogel_x, directional_x) records the radiance emitted from hogel hogel_x toward direction directional_x.

# 3  The Integral Image and the Hogel

## 3.1  The Hogel Concept

A hogel (holographic element) is the elementary spatial unit of a holographic stereogram or light field display. Each hogel occupies a small region of the display surface and is responsible for emitting light at a set of specified directions toward the viewer. For a display with N hogels arranged horizontally, each of width p (the hogel pitch), the hogel at index h occupies the horizontal interval [h·p, (h+1)·p) on the display surface.

## 3.2  Horizontal Parallax Only (HPO)

A horizontal-parallax-only (HPO) display provides binocular disparity and motion parallax exclusively in the horizontal direction. In such a display, the directional resolution in the vertical dimension is 1, meaning the integral image stores only a single vertical direction per hogel. Vertical spatial resolution is preserved — different rows of the image correspond to different vertical positions on the display — but there is no vertical parallax between views. This is the operating mode of the display described by the configuration file used here, where directionalResolution = [81, 1].

## 3.3  Integral Image Layout

The integral image is a single raster image whose width equals the product of the hogel count N and the directional resolution D. Hogels are laid out horizontally with spatial index s as the outer (slow) dimension and directional index dx as the inner (fast) dimension. The global pixel column for hogel s at directional index dx is:

col = s * D + dx

This one-dimensional layout encodes the full two-dimensional light field L(s, dx) in a single image, where rows correspond to vertical spatial positions (unchanged between views in HPO mode). In vector notation this is written as:

curPixel.x = spatialPos.x * directional.x + directionalPos.x

where directional.x = D (pixels per hogel), spatialPos.x = s is the hogel index, and directionalPos.x = dx is the directional index within the hogel.

# 4  Directional Index to Ray Angle Mapping

Each directional index dx within a hogel corresponds to a specific outgoing ray direction. The display has a full horizontal field of view of FOV degrees; the mapping uses the tangent of the half-angle:

maxAngle = tan(FOV / 2)

The use of tangent rather than angle is consistent with the standard pinhole camera perspective-projection model, where pixel positions are proportional to tan(theta), not theta itself. The full directional axis spans the range [0, D-1], mapped linearly to tangent-space angles:

tan(theta_dx) = (1 - 2*dx / (D-1)) * maxAngle

Inverting: directional index dx = 0 corresponds to the most rightward ray (positive tan(theta)), and dx = D-1 corresponds to the most leftward ray. The centre index dx = (D-1)/2 corresponds to the straight-ahead direction (theta = 0).

Because dx maps linearly to tan(theta) (not to theta itself), equal pixel steps do not correspond to equal angular increments. Near the axis the angular step is largest; near the edges it is smallest. This is a direct consequence of the tangent nonlinearity in the perspective model.

For the present display: FOV = 60 deg, D = 81, so maxAngle = tan(30 deg) = 0.5774. The angular step per pixel near the axis is approximately 2 * maxAngle / (D - 1) = 0.01444 rad/pixel = 0.827 deg/pixel.

# 5  The Assembled View and Its Orthographic Character

The simplest way to construct a quilt tile for view index v is to sample each hogel at the same directional index dx = v. The resulting image has one pixel per hogel:

quilt_pixel[v, h] = integral[ :, h*D + v ]       (assembled view)

This corresponds to placing the camera plane at infinity: every hogel emits light in the same parallel direction. In the two-plane light field framework, an orthographic view is produced when the (u, v) camera plane lies at infinity and the directional coordinate is fixed. A key property of orthographic projection is that parallel edges of an object remain parallel and that objects at different depths appear at the same apparent width. Consequently, the near edge and far edge of a rectangular table appear at identical horizontal extents in an assembled-view quilt tile — irrespective of which directional index v is sampled — because the number of hogels covering each edge equals its physical width divided by the hogel pitch, independent of depth.

Levoy and Hanrahan explicitly distinguish orthographic and perspective extraction from a light field: orthographic extraction fixes the direction and varies the camera position; perspective extraction fixes the camera position and varies the direction. The assembled-view approach performs orthographic extraction. For a light field display whose quilt tiles are expected to represent perspective views from specific camera positions, the assembled view is therefore incorrect, producing a flat rendering without true perspective foreshortening.

# 6  Perspective-Correct View Synthesis

## 6.1  Virtual Camera Setup

Each quilt tile v (where v ranges from 0 to D-1) represents a perspective view from a distinct camera position. The camera for tile v corresponds to the directional angle theta_v defined by the same mapping used for hogel emission:

tan(theta_v) = (1 - 2*v / (D-1)) * maxAngle

The camera is positioned on the camera plane at perpendicular distance z_cam from the display surface, horizontally displaced so that the straight-ahead ray from the camera aligns with direction theta_v:

x_cam(v) = display_center + z_cam * tan(theta_v)

where display_center = (N-1)/2 is the centre hogel index. When v = (D-1)/2 (the centre view), the camera is directly in front of the display centre.

## 6.2  Ray–Display Intersection and Hogel Selection

In the standard pinhole camera model, a ray from camera position (x_cam, z_cam) through pixel position px strikes the display plane at:

x_display(px) = px                    (when tile_width = N, one pixel per hogel)

The hogel index is simply h = px when the tile width equals the hogel count N, as chosen in integral_to_quilt_v3.py. For a tile of arbitrary width W, the display position would be x_display = px * N / W and the hogel index h = floor(x_display).

## 6.3  Direction of the Ray at the Hogel

The ray from camera (x_cam, z_cam) to hogel (h, 0) has direction vector (h - x_cam, -z_cam). The horizontal tangent of the ray angle (measured from the display normal) at hogel h is:

tan(theta_ray) = (h - x_cam) / z_cam   (direction of ray arriving at hogel h)

Substituting x_cam = display_center + z_cam * tan(theta_v):

tan(theta_ray) = (h - display_center) / z_cam - tan(theta_v)

## 6.4  Mapping Ray Angle to Directional Index

The display convention maps a ray tangent to a normalised angle variable angles.x using the negated tangent divided by maxAngle, then shifted to [0, 1]:

angles.x = -(dir.x / dir.z) / maxAngle

angles.x = (angles.x / 2) + 0.5

Here dir.x / dir.z is the ray slope, which equals -tan(theta_ray) (the negative sign arises because the ray travels from camera toward display, i.e., in the negative z direction). Substituting:

angles.x = tan(theta_ray) / maxAngle

angles.x_normalised = angles.x / 2 + 0.5

The directional index is then:

dx = (D - 1) * angles.x_normalised

= (D - 1) * (tan(theta_ray) / (2 * maxAngle) + 0.5)

## 6.5  Closed-Form Per-Hogel Directional Index

Substituting the expression for tan(theta_ray) from Section 6.3 and simplifying:

dx(h, v) = (D-1) * [ ((h - display_center)/z_cam - tan(theta_v)) / (2*maxAngle) + 0.5 ]

Expanding using tan(theta_v) = (1 - 2*v/(D-1)) * maxAngle:

dx(h, v) = (D-1)/2 - (D-1)*(h - display_center)/(2*z_cam*maxAngle)

+ (D-1)/2 * (1 - (1 - 2v/(D-1)))

After algebraic simplification this reduces to the final result:

dx(h, v)  =  v  +  (D-1) * (h - display_center) / (2 * z_cam * maxAngle)

This is the central formula of the algorithm. It states that the directional index assigned to hogel h in tile v equals the tile index v plus a parallax offset that grows linearly with the hogel's distance from the display centre. Hogels to the right of centre (h > display_center) receive a higher directional index (more leftward direction) in each tile; hogels to the left receive a lower index (more rightward direction). This is precisely the perspective parallax: a nearby object in front of the display casts rays that diverge more rapidly across hogels than a distant object.

The assembled view of Section 5 is recovered as the limit z_cam -> infinity, which drives the parallax offset to zero and yields dx(h, v) = v for all hogels — confirming the orthographic interpretation.

# 7  The Natural Viewing Distance

The camera distance z_cam is a free parameter that controls the strength of perspective. Setting it to the natural viewing distance — the distance at which the camera field of view exactly matches the display directional field of view — yields a convenient normalisation.

The camera field of view equals the display FOV when the half-width of the display subtends the half-angle maxAngle = tan(FOV/2) at the camera:

tan(FOV/2) = (N/2 * p) / z_cam_physical

=> z_cam_physical = N*p / (2 * maxAngle)

Expressing z_cam in hogel units (dividing by the hogel pitch p):

z_cam = N / (2 * maxAngle)

Substituting into the closed-form formula:

dx(h, v)  =  v  +  (D-1) * (h - display_center) / N        (natural z_cam)

This is the default used in integral_to_quilt_v3.py. For the present display (N = 819, D = 81, maxAngle = 0.5774):

z_cam  =  819 / (2 * 0.5774)  =  709 hogels

parallax coefficient  =  (D-1) / N  =  80 / 819  =  0.0977 dx-steps per hogel

This means that at the rightmost hogel (h = 818), the directional index is shifted by 0.0977 * 408.5 ≈ +40 steps relative to the tile index v; at the leftmost hogel it is shifted by -40 steps. The centre view (v = 40) therefore sweeps the full directional range [0, 80] from left to right hogel, which is geometrically consistent with a camera placed exactly at the centre of the display.

# 8  Handling Out-of-Range Indices

For extreme view indices (v near 0 or D-1), the computed dx(h, v) may fall outside [0, D-1] for hogels near the opposite edge of the display. This occurs because the virtual camera is positioned outside the directional FOV of those hogels: the angle from the edge hogel to the far-displaced camera exceeds maxAngle. Physically, those hogels cannot record that view direction.

The implementation clamps out-of-range indices to the nearest valid value:

dx_clamped = clip(round(dx(h, v)), 0, D-1)

Clamping assigns the edge directional sample to hogels beyond the angular limit. This is equivalent to assuming the light field is constant beyond its recorded angular extent — a standard boundary-fill assumption used in image resampling.

# 9  Algorithm Summary

Given the integral image I[row, col] and parameters N, D, maxAngle, z_cam, the quilt is computed as follows:

- Compute the parallax coefficient: k = (D-1) / (2 * z_cam * maxAngle)

- For each tile v in [0, D-1]:

- For each hogel h in [0, N-1]:  compute dx = round(v + k*(h - (N-1)/2)), clamped to [0, D-1]

- Sample all rows of column h*D + dx from the integral image and write into quilt column v*N + h.

The result is a quilt image of width N * D (equal to the integral image width) and the same height as the integral image, containing D perspective tiles each N pixels wide.

# 10  Display Configuration Parameters

The following table summarises all parameters derived from the display configuration JSON file used in this implementation.

| Parameter | Symbol | Value (config) | Description |
| --- | --- | --- | --- |
| Hogel count | N | 819 | Horizontal spatial positions (spatialResolution[0]) |
| Directional resolution | D | 81 | Views per hogel (directionalResolution[0]) |
| Display FOV | FOV | 60 deg | Full horizontal angular span (displayFOV[0]) |
| Maximum angle | maxAngle | tan(30 deg) = 0.5774 | Half-FOV tangent; maps directional pixels to ray angles |
| Hogel pitch | p | 13.656 / 819 = 0.01667 units | Physical width of one hogel (capturePlaneDimensions[0] / N) |
| Natural camera distance | z_cam | N / (2 * maxAngle) = 709 hogels | Distance at which camera FOV equals display FOV; default in v3 |
| Display center | c | (N-1)/2 = 409 | Centre hogel index, used as the origin of parallax |

# References

[1]  Adelson, E.H. and Bergen, J.R. (1991). The Plenoptic Function and the Elements of Early Vision. In Landy, M. and Movshon, J.A. (eds.), Computation Models of Visual Processing. MIT Press, Cambridge, MA.

[2]  Levoy, M. and Hanrahan, P. (1996). Light Field Rendering. Proceedings of ACM SIGGRAPH '96, pp. 31–42. https://graphics.stanford.edu/papers/light/

[3]  Gortler, S.J., Grzeszczuk, R., Szeliski, R., and Cohen, M.F. (1996). The Lumigraph. Proceedings of ACM SIGGRAPH '96, pp. 43–54.

[4]  Gershun, A. (1936). The Light Field. Moscow, 1936. Translated by P. Moon and G. Timoshenko in Journal of Mathematics and Physics, Vol. XVIII, MIT, 1939, pp. 51–151.

[5]  Benton, S.A. (1983). Survey of Holographic Stereograms. Processing and Display of Three-Dimensional Data, Proc. SPIE, Vol. 367, pp. 15–19.

[6]  Halle, M.W. (1994). Holographic Stereograms as Discrete Imaging Systems. Practical Holography VIII, Proc. SPIE, Vol. 2176, pp. 73–84.

[7]  Foley, J.D., van Dam, A., Feiner, S.K., and Hughes, J.F. (1995). Computer Graphics: Principles and Practice, 2nd ed. Addison-Wesley, Reading, MA. Chapter 6 (Viewing in 3D).

[8]  Shirley, P. and Marschner, S.R. (2009). Fundamentals of Computer Graphics, 3rd ed. A K Peters, Wellesley, MA. Chapter 4 (Ray Tracing) and Chapter 7 (Viewing).

Adelson, E.H. and Bergen, J.R. (1991). The Plenoptic Function and the Elements of Early Vision. In Landy, M. and Movshon, J.A. (eds.), Computation Models of Visual Processing. MIT Press, Cambridge, MA.

Levoy, M. and Hanrahan, P. (1996). Light Field Rendering. Proceedings of ACM SIGGRAPH '96, New Orleans, LA, pp. 31-42. https://graphics.stanford.edu/papers/light/

Gershun, A. (1936). The Light Field. Moscow, 1936. Translated by P. Moon and G. Timoshenko in Journal of Mathematics and Physics, Vol. XVIII, MIT, 1939, pp. 51-151.

Gortler, S.J., Grzeszczuk, R., Szeliski, R., and Cohen, M.F. (1996). The Lumigraph. Proceedings of ACM SIGGRAPH '96, New Orleans, LA, pp. 43-54.

Benton, S.A. (1983). Survey of Holographic Stereograms. Processing and Display of Three-Dimensional Data, Proc. SPIE, Vol. 367, pp. 15-19.

Foley, J.D., van Dam, A., Feiner, S.K., and Hughes, J.F. (1995). Computer Graphics: Principles and Practice, 2nd ed. Addison-Wesley, Reading, MA. Chapter 6 (Viewing in 3D).

Shirley, P. and Marschner, S.R. (2009). Fundamentals of Computer Graphics, 3rd ed. A K Peters, Wellesley, MA. Chapter 4 (Ray Tracing) and Chapter 7 (Viewing).